Guidelines:

- Work with your assigned teams to complete tasks 1 through 3 in class today

- Complete tasks 4-6 on your own (with appropriate help from your teammates)

- Each student will turn in their own work

- The objective is to design and simulate a speed controller for your unloaded DC motor

Project Description:

Next week we will be controlling your unloaded DC motor. To set this up, we need to design a control strategy and verify that it works in simulation.

Tasks:

- Finalize your transfer function for your DC motor

  - Recall that the transfer function for the DC motor is:

ΩsEins=KJs+bLas+Ra+KKb

  - Record your best values for the transfer function parameters in the table below:

| Parameter (Units) | Typical Value | Best Source | Your Value |
| --- | --- | --- | --- |
| Ra Ω | 9.2 | Project 1 – Task 2a |  |
| La H | 0.2110-3 | Project 1 – Task 2c |  |
| K N mA | 0.014 | Project 1 – Task 5c |  |
| KbV srad | 0.014 | Project 1 – Task 5c |  |
| b N m s | 2.510-7 | Project 2 – Task 2c |  |
| J kg m2 | 110-6→310-6 | Project 3 – Task 4c |  |

  - Write the transfer function with your numbers below.

- Explore the open-loop control strategy

  - For open-loop control, there is no feedback loop. The transfer function of the plant G=YU is the same as the transfer function for the system T=YR because the reference input (R) equals the plant input (U).

  - With this control strategy, use the final value theorem to find the final speed for a unit step voltage input on the transfer function.

  - You could run this model at any desired speed just by changing the input voltage. What voltage would you supply so that the mathematical model predicts a motor speed of 500 rad/s (4775 rpm)?

  - Explain why this open-loop strategy is not a good idea for a real DC motor where the load might change

- Consider a proportional controller with a feedback loop

  - Write a block diagram for a controller that includes a unity feedback path. The microcontroller we used for the last project can be programmed to measure shaft speed from the encoder and compare this to a desired reference input speed. This means the forward path of your block will need to have: a block for the controller Gc; and a block for the motor plant G.

  - Using proportional control Gc=k calculate the steady-state error of this control scheme for a reference step input of rt=100rads. This control block converts the input speed error rads to an output that is the input voltage to the motor V, so k needs to have units. Leave ess as a function of k for now.

  - What gain would be required for this type 0 system to have a 1% error to a step input?

  - Calculate the pole locations of the resulting controlled systems with your gain value (k). Do these pole locations indicate a stable system?

  - Using your calculated gain value k, what control signal will initially be calculated when the reference input is stepped up to rt=300rads while the output is still at rest? Hint: your answer should just be uinit=3k.

  - If you have correctly calculated parts d) and e), at least one of these answers should indicate why this is not an acceptable control strategy. Describe what the problem is.

- The next step will be to try a PI controller. You will need some Matlab tools to select the parameters of this controller.

  - A generic PI controller can be written in two different forms in the Laplace domain, we will use both.

    - GPIs=kp+kis

    - GPIs=Ks+zis

  - There are three design criteria for this controller based on a reference step input of rt=300 rad/s

    - Steady-state error to a step input should be: ess<1%

    - The maximum input voltage to the plant should be: umax<10 V

    - The system should respond as quickly as is practical (ωc=100rads)

  - Create a Bode plot of the plant transfer function without the controller

  - Choose some appropriate values for K and zi and add the PI controller to the rest of the system for the Bode plot. Play around with values until you meet the desired crossover frequency.

  - Convert your standard form K and zi values to parameters of kp and ki

  - Calculate the initial command signal for your chosen values based on the formula uinit=kp300rads. The ki value does not matter because at time zero nothing has been integrated yet.

  - Based on what you learned from part e), find some new K and zi values that will meet the second design criteria (although they may not meet the third). You will need your results from part c) later, so don’t delete them.

- Simulate the response of your designed to control to verify that it works

  - Use the feedback command to find the closed-loop transfer function T

  - Use the step command with the structure y,t=step(3*T), which will store the values for a magnitude 3 step command without making a plot

  - Use the lsim command with the structure y = lsimGc,3-y,t, which will calculate the output of the control block based on the results of the step command

  - Make your own plot of the step results and verify that you meet the steady-state error criteria

  - Make your own plot of the lsim results and verify that you meet the maximum input voltage criteria at all times (not just at the initial time)

  - Modify control parameters if needed, then add your final Bode plot to your result from task 4c)

  - Graphically demonstrate your final controller by making a root locus plot of the entire forward path (Gc*G*.01). Add the poles of T with plus signs onto this graph using plotrealpoleT,imagpoleT,'+','MarkerSize',7

- Submit the following by the start of class on the due date

  - This worksheet with tasks 1 through 3 filled out

  - A published version of your Matlab script including

    - A Bode plot with three lines representing Tasks 4c, 4d, and 5f

    - A plot based on Task 5d

    - A plot based on Task 5e

    - A root locus plot based on Task 5g

    - For all of these plots, use a logical title (which cannot be the default for the plot), axis labels, and a legend where appropriate

    - The final controller transfer function in both standard forms

  - Also submit your Matlab script as a *.m file to D2L for originality checking